|  | ACETIC ACID GLACIAL EXTRA PURE Safety Data Sheet according to Regulation (EC) No. 1907/2006 (REACH) with its amendment Regulation (EU) 2015/830 Reference number: 00004 |
| --- | --- |

Issue date: 03-08-2022   Revision date: 03-08-2022   Supersedes version of: 02-10-2017   Version: 1.0

SECTION 1: Identification of the substance/mixture and of the company/undertaking

| 1.1. Product identifier. | :Substance :ACETIC ACID GLACIAL EXTRA PURE :607-002-00-6 :200-580-7 :64-19-7 :00004 :Acids :C2H4O2 |  |
| --- | --- | --- |
| Product form Trade name EC Index-No. EC-No. CAS-No. Product code Type of product Formula Chemical structure Synonyms |  |  |
|  | : |  |
|  | :Ethanoic acid, Methanecarboxylic acid |  |

1.2. Relevant identified uses of the substance or mixture and uses advised against

1.2.1. Relevant identified uses

Main use category 	:Industrial use,Professional use Industrial/Professional use spec 	:Industrial 
		For professional use only Use of the substance/mixture 	:Laboratory chemicals 
		Manufacture of substances 		Solvents

1.2.2. Uses advised against

No additional information available

1.3. Details of the supplier of the safety data sheet

LOBA CHEMIE PVT.LTD.

107 Wode House Road, Jehangir Villa, Colaba 
400005 Mumbai - INDIA 
T +91 22 6663 6663 - F +91 22 6663 6699 
info@lobachemie.com - www.lobachemie.com

| 1.4. Emergency telephone number |  |
| --- | --- |
| Emergency number | :+ 91 22 6663 6663 (9:00am - 6:00 pm) |

SECTION 2: Hazards identification

2.1. Classification of the substance or mixture

Classification according to Regulation (EC) No. 1272/2008 [CLP]

Flammable liquids, Category 3 	H226

Skin corrosion/irritation, Category 1, Sub-Category 1A 	H314

Full text of H-statements: see section 16

Adverse physicochemical, human health and environmental effects

Flammable liquid and vapour. Causes severe skin burns and eye damage.

| 03-08-2022 (Revision date) | EN (English) | 1/11 |
| --- | --- | --- |

ACETIC ACID GLACIAL EXTRA PURE

Safety Data Sheet 
according to Regulation (EC) No. 1907/2006 (REACH) with its amendment Regulation (EU) 2015/830

2.2. Label elements

Labelling according to Regulation (EC) No. 1272/2008 [CLP]

| Hazard pictograms (CLP) Signal word (CLP) Hazard statements (CLP) | : |  |  |
| --- | --- | --- | --- |
|  | GHS02 |  | GHS05 |
|  | :Danger :H226 - Flammable liquid and vapour. |  |  |

H314 - Causes severe skin burns and eye damage.

Precautionary statements (CLP) 	:P280 - Wear protective clothing, eye protection, face protection, protective gloves.

P305+P351+P338 - IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

P310 - Immediately call a doctor.

2.3. Other hazards

No additional information available

SECTION 3: Composition/information on ingredients

| 3.1. Substances |  |
| --- | --- |
| Substance type Name CAS-No. EC-No. EC Index-No. | :Mono-constituent :ACETIC ACID GLACIAL :64-19-7 :200-580-7 :607-002-00-6 |

3.2. Mixtures

Not applicable

SECTION 4: First aid measures

| 4.1. Description of first aid measures |  |
| --- | --- |
| First-aid measures general First-aid measures after inhalation | :Call a physician immediately. :Remove person to fresh air and keep comfortable for breathing. Give oxygen or artificial |

respiration if necessary. If you feel unwell, seek medical advice.

First-aid measures after skin contact 	:Gently wash with plenty of soap and water. Rinse skin with water/shower. Take off 		immediately all contaminated clothing. Call a physician immediately.

First-aid measures after eye contact 	:Remove contact lenses, if present and easy to do. Continue rinsing. Rinse cautiously with 		water for several minutes. Call a physician immediately.

First-aid measures after ingestion 	:Rinse mouth out with water. Do not induce vomiting because of corrosive effects. If you feel 		unwell, seek medical advice. Rinse mouth. Do not induce vomiting. Call a physician 		immediately.

| 4.2. Most important symptoms and effects, both acute and delayed |  |  |
| --- | --- | --- |
| Symptoms/effects | :Causes severe skin burns and eye damage. |  |
| Symptoms/effects after skin contact |  | :Burns. |
| Symptoms/effects after eye contact | :Causes serious eye damage. Serious damage to eyes. |  |
| Symptoms/effects after ingestion | :Burns. |  |

4.3. Indication of any immediate medical attention and special treatment needed

Treat symptomatically.

| 03-08-2022 (Revision date) | EN (English) | 2/11 |
| --- | --- | --- |

ACETIC ACID GLACIAL EXTRA PURE

Safety Data Sheet 
according to Regulation (EC) No. 1907/2006 (REACH) with its amendment Regulation (EU) 2015/830

SECTION 5: Firefighting measures

| 5.1. Extinguishing media. |  |
| --- | --- |
| Suitable extinguishing media | :dry chemical powder, alcohol-resistant foam, carbon dioxide (CO2). Water spray. Dry |

powder. Foam. Carbon dioxide.

| Unsuitable extinguishing media | :Do not use a heavy water stream. |
| --- | --- |

| 5.2. Special hazards arising from the substance or mixture |  |
| --- | --- |
| Fire hazard | :Flammable liquid and vapour. |
| Explosion hazard | :May form flammable/explosive vapour-air mixture. |
| Hazardous decomposition products in case of fire | :Toxic fumes may be released. |

| 5.3. Advice for firefighters |  |
| --- | --- |
| Protection during firefighting | :Do not attempt to take action without suitable protective equipment. Self-contained |

breathing apparatus. Complete protective clothing.

SECTION 6: Accidental release measures

| 6.1. Personal precautions, protective equipment and emergency procedures |  |
| --- | --- |
| General measures | :Remove ignition sources. Use special care to avoid static electric charges. No open flames. |

No smoking.

6.1.1. For non-emergency personnel

Emergency procedures 	:Ventilate spillage area. Evacuate unnecessary personnel. No open flames, no sparks, and 		no smoking. Avoid contact with skin and eyes. Do not breathe dust, fume, gas, mist, spray, 		vapours.

6.1.2. For emergency responders

Protective equipment 	:Do not attempt to take action without suitable protective equipment. Use personal protective 		equipment as required. For further information refer to section 8: "Exposure 
		controls/personal protection".

| Emergency procedures | :Ventilate area. Stop release. |
| --- | --- |

6.2. Environmental precautions

Avoid release to the environment.

| 6.3. Methods and material for containment and cleaning up |  |
| --- | --- |
| Methods for cleaning up | :Take up liquid spill into absorbent material. Collect spillage. On land, sweep or shovel into |

suitable containers. Soak up spills with inert solids, such as clay or diatomaceous earth as soon as possible. Notify authorities if product enters sewers or public waters.

| Other information | :Dispose of materials or solid residues at an authorized site. |
| --- | --- |

6.4. Reference to other sections

For further information refer to section 13.

SECTION 7: Handling and storage

| 7.1. Precautions for safe handling |  |
| --- | --- |
| Additional hazards when processed Precautions for safe handling | :Handle empty containers with care because residual vapours are flammable. :Ensure good ventilation of the work station. No open flames. No smoking. Take |

precautionary measures against static discharge. Use only non-sparking tools. Do not breathe dust, fume, gas, mist, spray, vapours. Avoid contact during pregnancy/while nursing. Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. Ground/bond container and receiving equipment. Flammable vapours may accumulate in the container. Use explosion-proof equipment. Wear personal protective equipment. Avoid contact with skin and eyes.

| 03-08-2022 (Revision date) | EN (English) | 3/11 |
| --- | --- | --- |

ACETIC ACID GLACIAL EXTRA PURE Safety Data Sheet 
according to Regulation (EC) No. 1907/2006 (REACH) with its amendment Regulation (EU) 2015/830

| Hygiene measures | :Wash hands, forearms and face thoroughly after handling. Wash contaminated clothing |
| --- | --- |

before reuse. Do not eat, drink or smoke when using this product. Always wash hands after handling the product.

| 7.2. Conditions for safe storage, including any incompatibilities |  |
| --- | --- |
| Technical measures | :Ground/bond container and receiving equipment. |
| Storage conditions | :Store in original container. Keep container tightly closed. Store in a well-ventilated place. |

Keep cool. Store in a well-ventilated place. Keep cool. Store locked up.

| Incompatible materials | :Heat sources. |
| --- | --- |

7.3. Specific end use(s)

No additional information available

SECTION 8: Exposure controls/personal protection

8.1. Control parameters

8.1.1. National occupational exposure and biological limit values 
No additional information available 
8.1.2. Recommended monitoring procedures 
No additional information available 
8.1.3. Air contaminants formed 
No additional information available 
8.1.4. DNEL and PNEC 
No additional information available 
8.1.5. Control banding 
No additional information available

8.2. Exposure controls

8.2.1. Appropriate engineering controls 
Appropriate engineering controls: 
Ensure good ventilation of the work station.

8.2.2. Personal protection equipment 
Personal protective equipment symbol(s):

8.2.2.1. Eye and face protection 
Eye protection: 
Safety glasses 
8.2.2.2. Skin protection 
Hand protection: 
protective gloves 
8.2.2.3. Respiratory protection 
Respiratory protection: 
Wear appropriate mask 
8.2.2.4. Thermal hazards 
No additional information available

| 03-08-2022 (Revision date) | EN (English) | 4/11 |
| --- | --- | --- |

ACETIC ACID GLACIAL EXTRA PURE Safety Data Sheet 
according to Regulation (EC) No. 1907/2006 (REACH) with its amendment Regulation (EU) 2015/830

8.2.3. Environmental exposure controls 
Environmental exposure controls: 
Avoid release to the environment.

SECTION 9: Physical and chemical properties

| 9.1. Information on basic physical and chemical properties |  |
| --- | --- |
| Physical state | :Liquid |
| Appearance | :Clear liquid. |
| Molecular mass | :60.05 g/mol |
| Colour | :Colourless. |
| Odour | :strong, vinegar-like. |
| Odour threshold | :No data available |
| pH | :2.4 (1.0 M solution) |
| Relative evaporation rate (butylacetate=1) | :0.97 |
| Melting point | :16.2 °C |
| Freezing point | :No data available |
| Boiling point | :117 – 118 °C |
| Flash point | :40 °C |
| Auto-ignition temperature | :485 °C |
| Decomposition temperature | :No data available |
| Flammability (solid, gas) | :Flammable liquid and vapour. |
| Vapour pressure | :15.2 hPa at 20°C |
| Relative vapour density at 20 °C | :2.1 |
| Relative density | :No data available |
| Density | :1.049 g/cm³ |
| Solubility | :Water: Miscible in water |
| Partition coefficient n-octanol/water (Log Pow) | :-0.17 |
| Viscosity, kinematic | :1.163 mm²/s |
| Viscosity, dynamic | :1.22 cP |
| Explosive properties | :No data available |
| Oxidising properties | :No data available |
| Lower explosive limit (LEL) | :4 vol % |
| Upper explosive limit (UEL) | :19.9 vol % |

9.2. Other information

No additional information available

SECTION 10: Stability and reactivity

10.1. Reactivity

Flammable liquid and vapour.

10.2. Chemical stability

Flammable liquid and vapour. May form flammable/explosive vapour-air mixture.

10.3. Possibility of hazardous reactions

No dangerous reactions known under normal conditions of use.

10.4. Conditions to avoid

Open flame. Overheating. Direct sunlight. Heat. Sparks. Avoid contact with hot surfaces. No flames, no sparks. Eliminate all sources of ignition.

10.5. Incompatible materials

No additional information available

| 03-08-2022 (Revision date) | EN (English) | 5/11 |
| --- | --- | --- |

ACETIC ACID GLACIAL EXTRA PURE

Safety Data Sheet 
according to Regulation (EC) No. 1907/2006 (REACH) with its amendment Regulation (EU) 2015/830

10.6. Hazardous decomposition products

May release flammable gases. Thermal decomposition generates : Corrosive vapours.

SECTION 11: Toxicological information

| 11.1. Information on toxicological effects |  |
| --- | --- |
| Acute toxicity (oral) Acute toxicity (dermal) Acute toxicity (inhalation) Skin corrosion/irritation | :Not classified :Not classified :Not classified :Causes severe skin burns. |

pH: 2.4 (1.0 M solution) 
Serious eye damage/irritation 	:Assumed to cause serious eye damage 		pH: 2.4 (1.0 M solution)

| Respiratory or skin sensitisation Germ cell mutagenicity Carcinogenicity Reproductive toxicity STOT-single exposure STOT-repeated exposure Aspiration hazard | :Not classified :Not classified :Not classified :Not classified :Not classified :Not classified :Not classified |
| --- | --- |

SECTION 12: Ecological information

12.1. Toxicity

Ecology - general 	:Before neutralisation, the product may represent a danger to aquatic organisms. Hazardous to the aquatic environment, short–term 	:Not classified 
(acute) 
Hazardous to the aquatic environment, long–term 	:Not classified 
(chronic)

12.2. Persistence and degradability

No additional information available

12.3. Bioaccumulative potential

| ACETIC ACID GLACIAL EXTRA PURE (64-19-7) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | -0.17 |

12.4. Mobility in soil

No additional information available

12.5. Results of PBT and vPvB assessment

No additional information available

12.6. Other adverse effects

| No additional information available 03-08-2022 (Revision date) | EN (English) | 6/11 |
| --- | --- | --- |

ACETIC ACID GLACIAL EXTRA PURE Safety Data Sheet 
according to Regulation (EC) No. 1907/2006 (REACH) with its amendment Regulation (EU) 2015/830

SECTION 13: Disposal considerations

| 13.1. Waste treatment methods. |  |
| --- | --- |
| Waste treatment methods | :Dispose of contents/container in accordance with licensed collector’s sorting instructions. |

Product/Packaging disposal recommendations

:Dispose of contents/container to a licensed hazardous-waste disposal contractor or collection site except for empty clean containers which can be disposed of as non-hazardous waste.

Additional information 	:Handle empty containers with care because residual vapours are flammable. Flammable

vapours may accumulate in the container.

SECTION 14: Transport information

In accordance with ADR / IMDG / IATA / ADN / RID

| 14.1. UN number |  |
| --- | --- |
| UN-No. (ADR) UN-No. (IMDG) UN-No. (IATA) UN-No. (ADN) UN-No. (RID) | :UN 2789 :UN 2789 :UN 2789 :UN 2789 :UN 2789 |

| 14.2. UN proper shipping name |  |
| --- | --- |
| Proper Shipping Name (ADR) Proper Shipping Name (IMDG) Proper Shipping Name (IATA) Proper Shipping Name (ADN) Proper Shipping Name (RID) Transport document description (ADR) Transport document description (IMDG) Transport document description (IATA) Transport document description (ADN) Transport document description (RID) | :ACETIC ACID, GLACIAL :ACETIC ACID, GLACIAL :Acetic acid, glacial :ACETIC ACID, GLACIAL :ACETIC ACID, GLACIAL :UN 2789 ACETIC ACID, GLACIAL, 8 (3), II, (D/E) :UN 2789 ACETIC ACID, GLACIAL, 8 (3), II :UN 2789 Acetic acid, glacial, 8 (3), II :UN 2789 ACETIC ACID, GLACIAL, 8 (3), II :UN 2789 ACETIC ACID, GLACIAL, 8 (3), II |

14.3. Transport hazard class(es)

ADR

| Transport hazard class(es) (ADR) | :8 (3) |  |  |
| --- | --- | --- | --- |
| Danger labels (ADR) | :8, 3 |  |  |
|  | : |  |  |

IMDG

| Transport hazard class(es) (IMDG) | :8 (3) |  |  |
| --- | --- | --- | --- |
| Danger labels (IMDG) | :8, 3 |  |  |
|  | : |  |  |

IATA

| Transport hazard class(es) (IATA) | :8 (3) |  |  |
| --- | --- | --- | --- |
| Danger labels (IATA) | :8, 3 |  |  |
| 03-08-2022 (Revision date) |  | EN (English) | 7/11 |

ACETIC ACID GLACIAL EXTRA PURE

Safety Data Sheet 
according to Regulation (EC) No. 1907/2006 (REACH) with its amendment Regulation (EU) 2015/830

| : |  |  |
| --- | --- | --- |

ADN

| Transport hazard class(es) (ADN) | :8 (3) |  |  |
| --- | --- | --- | --- |
| Danger labels (ADN) | :8, 3 |  |  |
|  | : |  |  |

RID

| Transport hazard class(es) (RID) | :8 (3) |  |  |
| --- | --- | --- | --- |
| Danger labels (RID) | :8, 3 |  |  |
|  | : |  |  |

| 14.4. Packing group |  |
| --- | --- |
| Packing group (ADR) Packing group (IMDG) Packing group (IATA) Packing group (ADN) Packing group (RID) | :II :II :II :II :II |

| 14.5. Environmental hazards |  |
| --- | --- |
| Dangerous for the environment Marine pollutant Other information | :No :No :No supplementary information available |

14.6. Special precautions for user

Overland transport 
Classification code (ADR) 	:CF1 
Limited quantities (ADR) 	:1l 
Excepted quantities (ADR) 	:E2 
Packing instructions (ADR) 
	:P001, IBC02 Mixed packing provisions (ADR) 	:MP15 
Portable tank and bulk container instructions (ADR) 	:T7 
Portable tank and bulk container special provisions 	:TP2 
(ADR)

| Tank code (ADR) | :L4BN |  |  |
| --- | --- | --- | --- |
| Vehicle for tank carriage | :FL |  |  |
| Transport category (ADR) | :2 |  |  |
| Special provisions for carriage - Operation (ADR) | :S2 |  |  |
| Hazard identification number (Kemler No.) | :83 |  |  |
| Orange plates | : |  |  |
| Tunnel restriction code (ADR) | :D/E |  |  |
| EAC code | :•2P |  |  |
| APP code | :A(fl) |  |  |
| 03-08-2022 (Revision date) | EN (English) |  | 8/11 |

ACETIC ACID GLACIAL EXTRA PURE

Safety Data Sheet 
according to Regulation (EC) No. 1907/2006 (REACH) with its amendment Regulation (EU) 2015/830

Transport by sea 
Limited quantities (IMDG) 	:1 L 
Excepted quantities (IMDG) 	:E2 
Packing instructions (IMDG) 	:P001 
IBC packing instructions (IMDG) 
	:IBC02 Tank instructions (IMDG) 	:T7 
Tank special provisions (IMDG) 	:TP2 
EmS-No. (Fire) 	:F-E 
EmS-No. (Spillage) 
	:S-C Stowage category (IMDG) 	:A 
Segregation (IMDG) 	:SGG1, SG36, SG49 
Properties and observations (IMDG) 	:Colourless flammable liquid with a pungent odour. When pure, crystallizes below 16°C.

Flashpoint: 40°C c.c. (pure product) 60°C c.c. (80% solution) Explosive limits: 4% to 17%  Miscible with water. Corrosive to lead and most other metals. Corrosive to skin, eyes and mucous membranes.

MFAG-No 	:132

Air transport 
PCA Excepted quantities (IATA) 	:E2 
PCA Limited quantities (IATA) 	:Y840 
PCA limited quantity max net quantity (IATA) 	:0.5L 
PCA packing instructions (IATA) 
	:851 PCA max net quantity (IATA) 	:1L 
CAO packing instructions (IATA) 	:855 
CAO max net quantity (IATA) 	:30L 
ERG code (IATA) 	:8F

Inland waterway transport 
Classification code (ADN) 	:CF1 
Limited quantities (ADN) 	:1 L 
Excepted quantities (ADN) 	:E2 
Equipment required (ADN) 
	:PP, EP, EX, A Ventilation (ADN) 	:VE01 
Number of blue cones/lights (ADN) 	:1

Rail transport 
Classification code (RID) 	:CF1 
Limited quantities (RID) 	:1L 
Excepted quantities (RID) 	:E2 
Packing instructions (RID) 
	:P001, IBC02 Mixed packing provisions (RID) 	:MP15 
Portable tank and bulk container instructions (RID) 	:T7 
Portable tank and bulk container special provisions 	:TP2 
(RID)

| Tank codes for RID tanks (RID) Transport category (RID) Colis express (express parcels) (RID) Hazard identification number (RID) | :L4BN :2 :CE6 :83 |
| --- | --- |

14.7. Transport in bulk according to Annex II of Marpol and the IBC Code

Not applicable

| 03-08-2022 (Revision date) | EN (English) | 9/11 |
| --- | --- | --- |

ACETIC ACID GLACIAL EXTRA PURE

Safety Data Sheet 
according to Regulation (EC) No. 1907/2006 (REACH) with its amendment Regulation (EU) 2015/830

SECTION 15: Regulatory information

15.1. Safety, health and environmental regulations/legislation specific for the substance or mixture.

15.1.1. EU-Regulations

| EU restriction list (REACH Annex XVII) |  |
| --- | --- |
| Reference code | Applicable on |
| 3(a) | ACETIC ACID GLACIAL EXTRA PURE |
| 3(b) | ACETIC ACID GLACIAL EXTRA PURE |
| 40. | ACETIC ACID GLACIAL EXTRA PURE |

ACETIC ACID GLACIAL EXTRA PURE is not on the REACH Candidate List 
ACETIC ACID GLACIAL EXTRA PURE is not on the REACH Annex XIV List 
ACETIC ACID GLACIAL EXTRA PURE is not subject to Regulation (EU) No 649/2012 of the European Parliament and of the Council of 4 july 2012 concerning the export and import of hazardous chemicals.

ACETIC ACID GLACIAL EXTRA PURE is not subject to Regulation (EU) No 2019/1021 of the European Parliament and of the Council of 20 June 2019 on persistent organic pollutants

15.1.2. National regulations

Germany 
Water hazard class (WGK) 	:WGK 1, Slightly hazardous to water (Classification according to AwSV; ID No. 93)

Hazardous Incident Ordinance (12. BImSchV) Netherlands

:Is not subject of the Hazardous Incident Ordinance (12. BImSchV)

SZW-lijst van kankerverwekkende stoffen 	:The substance is not listed SZW-lijst van mutagene stoffen 	:The substance is not listed SZW-lijst van reprotoxische stoffen – Borstvoeding 	:The substance is not listed SZW-lijst van reprotoxische stoffen – 	:The substance is not listed Vruchtbaarheid 
SZW-lijst van reprotoxische stoffen – Ontwikkeling 	:The substance is not listed

Denmark 
Class for fire hazard 	:Class II-1 
Store unit 	:5 liter 
Classification remarks 	:R10 <H226;H314>; Emergency management guidelines for the storage of flammable liquids 		must be followed

| Danish National Regulations | :Young people below the age of 18 years are not allowed to use the product |
| --- | --- |

15.2. Chemical safety assessment

No chemical safety assessment has been carried out

SECTION 16: Other information Abbreviations and acronyms. ADN; European Agreement concerning the International Carriage of Dangerous Goods by Inland Waterways. ADR; European Agreement concerning the International Carriage of Dangerous Goods by Road. ATE; Acute Toxicity Estimate. BCF; Bioconcentration factor. BLV; Biological limit value. BOD; Biochemical oxygen demand (BOD). COD; Chemical oxygen demand (COD). DMEL; Derived Minimal Effect level. DNEL; Derived-No Effect Level. EC-No.; European Community number.

| 03-08-2022 (Revision date) | EN (English) | 10/11 |
| --- | --- | --- |

ACETIC ACID GLACIAL EXTRA PURE Safety Data Sheet 
according to Regulation (EC) No. 1907/2006 (REACH) with its amendment Regulation (EU) 2015/830

| Abbreviations and acronyms |  |
| --- | --- |
| EC50 | Median effective concentration |
| EN | European Standard |
| IARC | International Agency for Research on Cancer |
| IATA | International Air Transport Association |
| IMDG | International Maritime Dangerous Goods |
| LC50 | Median lethal concentration |
| LD50 | Median lethal dose |
| LOAEL | Lowest Observed Adverse Effect Level |
| NOAEC | No-Observed Adverse Effect Concentration |
| NOAEL | No-Observed Adverse Effect Level |
| NOEC | No-Observed Effect Concentration |
| OECD | Organisation for Economic Co-operation and Development |
| OEL | Occupational Exposure Limit |
| PBT | Persistent Bioaccumulative Toxic |
| PNEC | Predicted No-Effect Concentration |
| RID | Regulations concerning the International Carriage of Dangerous Goods by Rail |
| SDS | Safety Data Sheet |
| STP | Sewage treatment plant |
| ThOD | Theoretical oxygen demand (ThOD) |
| TLM | Median Tolerance Limit |
| VOC | Volatile Organic Compounds |
| CAS-No. | Chemical Abstract Service number |
| N.O.S. | Not Otherwise Specified |
| vPvB | Very Persistent and Very Bioaccumulative |
| ED | Endocrine disrupting properties |

| Full text of H- and EUH-statements |  |
| --- | --- |
| Flam. Liq. 3 | Flammable liquids, Category 3 |
| Skin Corr. 1A | Skin corrosion/irritation, Category 1, Sub-Category 1A |
| H226 | Flammable liquid and vapour. |
| H314 | Causes severe skin burns and eye damage. |

Safety Data Sheet (SDS), EU

This information is based on our current knowledge and is intended to describe the product for the purposes of health, safety and environmental requirements only. It should not therefore be construed as guaranteeing any specific property of the product.

| 03-08-2022 (Revision date) | EN (English) | 11/11 |
| --- | --- | --- |