|  | ACETONE EXTRA PURE Safety Data Sheet according to Regulation (EC) No. 1907/2006 (REACH) with its amendment Regulation (EU) 2015/830 Reference number: 00011 |
| --- | --- |

Issue date: 06-08-2021   Revision date: 06-08-2021   Supersedes version of: 07-02-2017   Version: 1.0

SECTION 1: Identification of the substance/mixture and of the company/undertaking

| 1.1. Product identifier. | :Substance :ACETONE EXTRA PURE :606-001-00-8 :200-662-2 :67-64-1 :00011 :C3H6O |  |
| --- | --- | --- |
| Product form Trade name EC Index-No. EC-No. CAS-No. Product code Formula Chemical structure Synonyms |  |  |
|  | : |  |
|  | :Propanone, Dimethyl ketone |  |

1.2. Relevant identified uses of the substance or mixture and uses advised against

1.2.1. Relevant identified uses

Industrial/Professional use spec 	:Industrial 
Use of the substance/mixture 	:Laboratory chemicals 		Manufacture of substances 		Pharmaceuticals

1.2.2. Uses advised against

No additional information available

1.3. Details of the supplier of the safety data sheet

LOBA CHEMIE PVT.LTD.

107 Wode House Road, Jehangir Villa, Colaba 
400005 Mumbai - INDIA 
T +91 22 6663 6663 - F +91 22 6663 6699 
info@lobachemie.com - www.lobachemie.com

| 1.4. Emergency telephone number |  |
| --- | --- |
| Emergency number | :+ 91 22 6663 6663 (9:00am - 6:00 pm) |

SECTION 2: Hazards identification

2.1. Classification of the substance or mixture

Classification according to Regulation (EC) No. 1272/2008 [CLP]

Flammable liquids, Category 2 	H225

Serious eye damage/eye irritation, Category 2 	H319

Specific target organ toxicity — Single exposure, Category 3, Narcosis 	H336

Full text of H-statements: see section 16

Adverse physicochemical, human health and environmental effects

Highly flammable liquid and vapour. May cause drowsiness or dizziness. Causes serious eye irritation.

| 06-08-2021 (Revision date) | EN (English) | 1/11 |
| --- | --- | --- |

ACETONE EXTRA PURE

Safety Data Sheet 
according to Regulation (EC) No. 1907/2006 (REACH) with its amendment Regulation (EU) 2015/830

2.2. Label elements

Labelling according to Regulation (EC) No. 1272/2008 [CLP]

| Hazard pictograms (CLP) Signal word (CLP) Hazard statements (CLP) | : |  |  |
| --- | --- | --- | --- |
|  | GHS02 |  | GHS07 |
|  | :Danger :H225 - Highly flammable liquid and vapour. |  |  |

H319 - Causes serious eye irritation.

H336 - May cause drowsiness or dizziness.

Precautionary statements (CLP) 	:P210 - Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. 		No smoking.

P261 - Avoid breathing dust, fume, gas, mist, spray, vapours.

P305+P351+P338 - IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

| EUH-statements | :EUH066 - Repeated exposure may cause skin dryness or cracking. |
| --- | --- |

2.3. Other hazards

No additional information available

SECTION 3: Composition/information on ingredients

| 3.1. Substances |  |
| --- | --- |
| Substance type Name CAS-No. EC-No. EC Index-No. | :Mono-constituent :ACETONE EXTRA PURE :67-64-1 :200-662-2 :606-001-00-8 |

3.2. Mixtures

Not applicable

SECTION 4: First aid measures

| 4.1. Description of first aid measures |  |
| --- | --- |
| First-aid measures general First-aid measures after inhalation | :Call a poison center or a doctor if you feel unwell. :Remove person to fresh air and keep comfortable for breathing. Remove person to fresh air |

and keep comfortable for breathing. Call a POISON CENTER/doctor if you feel unwell.

First-aid measures after skin contact 	:Repeated exposure may cause skin dryness or cracking. Rinse skin with water/shower. 		Take off immediately all contaminated clothing.

First-aid measures after eye contact 	:If eye irritation persists: Rinse immediately with plenty of water, Obtain medical attention if 		pain, blinking or redness persists. Get medical advice/attention. Rinse cautiously with water 		for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. If 		eye irritation persists: Get medical advice/attention.

First-aid measures after ingestion 	:Rinse mouth out with water. If you feel unwell, seek medical advice. Call a poison center or 		a doctor if you feel unwell.

| 4.2. Most important symptoms and effects, both acute and delayed |  |  |
| --- | --- | --- |
| Symptoms/effects | :May cause drowsiness or dizziness. |  |
| Symptoms/effects after inhalation | :May cause drowsiness or dizziness. |  |
| Symptoms/effects after skin contact |  | :Repeated exposure may cause skin dryness or cracking. |
| Symptoms/effects after eye contact | :Causes serious eye irritation. Eye irritation. |  |

4.3. Indication of any immediate medical attention and special treatment needed

Treat symptomatically.

| 06-08-2021 (Revision date) | EN (English) | 2/11 |
| --- | --- | --- |

ACETONE EXTRA PURE

Safety Data Sheet 
according to Regulation (EC) No. 1907/2006 (REACH) with its amendment Regulation (EU) 2015/830

SECTION 5: Firefighting measures

| 5.1. Extinguishing media. |  |
| --- | --- |
| Suitable extinguishing media | :dry chemical powder, alcohol-resistant foam, carbon dioxide (CO2). Water spray. Dry |

powder. Foam. Carbon dioxide.

| Unsuitable extinguishing media | :Do not use a heavy water stream. |
| --- | --- |

| 5.2. Special hazards arising from the substance or mixture |  |
| --- | --- |
| Fire hazard | :Highly flammable liquid and vapour. |
| Explosion hazard | :May form flammable/explosive vapour-air mixture. |
| Hazardous decomposition products in case of fire | :Toxic fumes may be released. |

| 5.3. Advice for firefighters |  |
| --- | --- |
| Protection during firefighting | :Do not attempt to take action without suitable protective equipment. Self-contained |

breathing apparatus. Complete protective clothing.

SECTION 6: Accidental release measures

| 6.1. Personal precautions, protective equipment and emergency procedures |  |
| --- | --- |
| General measures | :Remove ignition sources. Use special care to avoid static electric charges. No open flames. |

No smoking.

6.1.1. For non-emergency personnel

Emergency procedures 	:Ventilate spillage area. Evacuate unnecessary personnel. No open flames, no sparks, and 		no smoking. Avoid breathing dust, fume, gas, mist, spray, vapours. Avoid contact with skin 		and eyes.

6.1.2. For emergency responders

Protective equipment 	:Do not attempt to take action without suitable protective equipment. Avoid breathing 		Vapours. For further information refer to section 8: "Exposure controls/personal protection".

6.2. Environmental precautions

Avoid release to the environment.

| 6.3. Methods and material for containment and cleaning up |  |
| --- | --- |
| Methods for cleaning up | :Take up liquid spill into absorbent material. Collect spillage. On land, sweep or shovel into |

suitable containers. Soak up spills with inert solids, such as clay or diatomaceous earth as soon as possible. Notify authorities if product enters sewers or public waters.

| Other information | :Dispose of materials or solid residues at an authorized site. |
| --- | --- |

6.4. Reference to other sections

For further information refer to section 13.

SECTION 7: Handling and storage

| 7.1. Precautions for safe handling |  |
| --- | --- |
| Additional hazards when processed Precautions for safe handling | :Handle empty containers with care because residual vapours are flammable. :No open flames. No smoking. Keep away from heat, hot surfaces, sparks, open flames and |

other ignition sources. No smoking. Ground/bond container and receiving equipment. Use only non-sparking tools. Take precautionary measures against static discharge. Flammable vapours may accumulate in the container. Use explosion-proof equipment. Wear personal protective equipment. Use only outdoors or in a well-ventilated area. Avoid breathing dust, fume, gas, mist, spray, vapours. Avoid contact with skin and eyes.

Hygiene measures 	:Wash hands, forearms and face thoroughly after handling. Do not eat, drink or smoke when 		using this product. Always wash hands after handling the product.

| 06-08-2021 (Revision date) | EN (English) | 3/11 |
| --- | --- | --- |

ACETONE EXTRA PURE 
Safety Data Sheet 
according to Regulation (EC) No. 1907/2006 (REACH) with its amendment Regulation (EU) 2015/830

| 7.2. Conditions for safe storage, including any incompatibilities |  |
| --- | --- |
| Technical measures | :Proper grounding procedures to avoid static electricity should be followed. Use explosion- |

proof electrical, lighting, ventilating equipment. Ground/bond container and receiving equipment.

Storage conditions 	:Keep in fireproof place. Store in a well-ventilated place. Keep cool. Keep container tightly 		closed. Store locked up.

| Incompatible materials | :Heat sources. |
| --- | --- |

7.3. Specific end use(s)

No additional information available

SECTION 8: Exposure controls/personal protection

8.1. Control parameters

8.1.1. National occupational exposure and biological limit values 
No additional information available

8.1.2. Recommended monitoring procedures 
No additional information available

8.1.3. Air contaminants formed 
No additional information available

8.1.4. DNEL and PNEC 
No additional information available

8.1.5. Control banding 
No additional information available

8.2. Exposure controls

8.2.1. Appropriate engineering controls

Appropriate engineering controls: 
Ensure good ventilation of the work station.

8.2.2. Personal protection equipment

Personal protective equipment: 
Protective clothing.

Personal protective equipment symbol(s):

8.2.2.1. Eye and face protection

Eye protection: 
Chemical goggles or safety glasses. Safety glasses

8.2.2.2. Skin protection 
No additional information available

8.2.2.3. Respiratory protection

Respiratory protection: 
Wear appropriate mask

8.2.2.4. Thermal hazards 
No additional information available

| 06-08-2021 (Revision date) | EN (English) | 4/11 |
| --- | --- | --- |

ACETONE EXTRA PURE 
Safety Data Sheet 
according to Regulation (EC) No. 1907/2006 (REACH) with its amendment Regulation (EU) 2015/830

8.2.3. Environmental exposure controls 
Environmental exposure controls: 
Avoid release to the environment.

SECTION 9: Physical and chemical properties

| 9.1. Information on basic physical and chemical properties |  |
| --- | --- |
| Physical state | :Liquid |
| Appearance | :Clear, colorless, volatile liquid. |
| Molecular mass | :< 58.08 g/mol |
| Colour | :Clear Colorless. |
| Odour | :fragrant, mint-like. |
| Odour threshold | :0.1 ppm |
| pH | :7 |
| Relative evaporation rate (butylacetate=1) | :No data available |
| Melting point | :-95.35 °C |
| Freezing point | :No data available |
| Boiling point | :56.2 °C |
| Flash point | :-20 °C |
| Critical temperature | :235 °C |
| Auto-ignition temperature | :465 °C |
| Decomposition temperature | :No data available |
| Flammability (solid, gas) | :Extremely flammable |

Highly flammable liquid and vapour.

| Vapour pressure Relative vapour density at 20 °C Relative density Density Relative gas density Solubility Partition coefficient n-octanol/water (Log Pow) Viscosity, kinematic Viscosity, dynamic Explosive properties Oxidising properties Lower explosive limit (LEL) Upper explosive limit (UEL) | :24 kPa :2 :No data available :0.79 g/cm³ :2 :Water: Complete :No data available :0.405 mm²/s :0.32 mPa·s (20°C) :No data available :No data available :2 vol % :13 vol % |
| --- | --- |

9.2. Other information

No additional information available

SECTION 10: Stability and reactivity

10.1. Reactivity

Highly flammable liquid and vapour.

10.2. Chemical stability

Highly flammable liquid and vapour. May form flammable/explosive vapour-air mixture.

10.3. Possibility of hazardous reactions

No dangerous reactions known under normal conditions of use.

10.4. Conditions to avoid

Open flame. Direct sunlight. Avoid contact with hot surfaces. Heat. No flames, no sparks. Eliminate all sources of ignition.

10.5. Incompatible materials

No additional information available

| 06-08-2021 (Revision date) | EN (English) | 5/11 |
| --- | --- | --- |

ACETONE EXTRA PURE

Safety Data Sheet 
according to Regulation (EC) No. 1907/2006 (REACH) with its amendment Regulation (EU) 2015/830

10.6. Hazardous decomposition products

May release flammable gases.

SECTION 11: Toxicological information

| 11.1. Information on toxicological effects |  |
| --- | --- |
| Acute toxicity (oral) Acute toxicity (dermal) Acute toxicity (inhalation) Skin corrosion/irritation | :Not classified :Not classified :Not classified :Not classified |

pH: 7 
Additional information 	:Repeated exposure may cause skin dryness or cracking. Serious eye damage/irritation 	:Causes serious eye irritation.

pH: 7

| Respiratory or skin sensitisation Germ cell mutagenicity Carcinogenicity Reproductive toxicity STOT-single exposure STOT-repeated exposure Aspiration hazard | :Not classified :Not classified :Not classified :Not classified :May cause drowsiness or dizziness. :Not classified :Not classified |
| --- | --- |

SECTION 12: Ecological information

12.1. Toxicity

| Ecology - general Hazardous to the aquatic environment, short-term (acute) Hazardous to the aquatic environment, long-term (chronic) No additional information available No additional information available No additional information available No additional information available No additional information available 13.1. Waste treatment methods. | :The product is not considered harmful to aquatic organisms nor to cause long-term adverse effects in the environment. erm rm; :Not classified :Not classified. :Dispose of contents/container in accordance with licensed collector’s sorting instructions. |
| --- | --- |
| Waste treatment methods |  |  |
| 06-08-2021 (Revision date) | EN (English) | 6/11 |

12.5. Results of PBT and vPvB assessment

12.6. Other adverse effects

SECTION 13: Disposal considerations

12.2. Persistence and degradability

12.3. Bioaccumulative potential

12.4. Mobility in soil

ACETONE EXTRA PURE 
Safety Data Sheet 
according to Regulation (EC) No. 1907/2006 (REACH) with its amendment Regulation (EU) 2015/830

Product/Packaging disposal recommendations

:Dispose of contents/container to a licensed hazardous-waste disposal contractor or collection site except for empty clean containers which can be disposed of as non-hazardous waste.

Additional information 	:Handle empty containers with care because residual vapours are flammable. Flammable

vapours may accumulate in the container.

SECTION 14: Transport information

In accordance with ADR / IMDG / IATA / ADN / RID

| 14.1. UN number |  |
| --- | --- |
| UN-No. (ADR) UN-No. (IMDG) UN-No. (IATA) UN-No. (ADN) UN-No. (RID) | :UN 1090 :UN 1090 :UN 1090 :UN 1090 :UN 1090 |

| 14.2. UN proper shipping name |  |
| --- | --- |
| Proper Shipping Name (ADR) Proper Shipping Name (IMDG) Proper Shipping Name (IATA) Proper Shipping Name (ADN) Proper Shipping Name (RID) Transport document description (ADR) Transport document description (IMDG) Transport document description (IATA) Transport document description (ADN) Transport document description (RID) | :ACETONE :ACETONE :Acetone :ACETONE :ACETONE :UN 1090 ACETONE, 3, II, (D/E) :UN 1090 ACETONE, 3, II (-20°C c.c.) :UN 1090 Acetone, 3, II :UN 1090 ACETONE, 3, II :UN 1090 ACETONE, 3, II |

14.3. Transport hazard class(es)

ADR

| Transport hazard class(es) (ADR) Danger labels (ADR) | :3 :3 |  |
| --- | --- | --- |
|  | : |  |

IMDG

| Transport hazard class(es) (IMDG) Danger labels (IMDG) | :3 :3 |  |
| --- | --- | --- |
|  | : |  |

IATA

| Transport hazard class(es) (IATA) | :3 |  |  |
| --- | --- | --- | --- |
| Danger labels (IATA) | :3 |  |  |
|  | : |  |  |
| 06-08-2021 (Revision date) | EN (English) |  | 7/11 |

ACETONE EXTRA PURE

Safety Data Sheet 
according to Regulation (EC) No. 1907/2006 (REACH) with its amendment Regulation (EU) 2015/830

ADN

| Transport hazard class(es) (ADN) Danger labels (ADN) | :3 :3 |  |
| --- | --- | --- |
|  | : |  |

RID

| Transport hazard class(es) (RID) Danger labels (RID) | :3 :3 |  |
| --- | --- | --- |
|  | : |  |

| 14.4. Packing group |  |
| --- | --- |
| Packing group (ADR) Packing group (IMDG) Packing group (IATA) Packing group (ADN) Packing group (RID) | :II :II :II :II :II |

| 14.5. Environmental hazards |  |
| --- | --- |
| Dangerous for the environment Marine pollutant Other information | :No :No :No supplementary information available |

14.6. Special precautions for user

Overland transport 
Classification code (ADR) 	:F1 
Limited quantities (ADR) 	:1l 
Excepted quantities (ADR) 	:E2 
Packing instructions (ADR) 
	:P001, IBC02, R001 Mixed packing provisions (ADR) 	:MP19 
Portable tank and bulk container instructions (ADR) 	:T4 
Portable tank and bulk container special provisions 	:TP1 
(ADR)

| Tank code (ADR) Vehicle for tank carriage Transport category (ADR) Special provisions for carriage - Operation (ADR) Hazard identification number (Kemler No.) Orange plates Tunnel restriction code (ADR) EAC code | :LGBF :FL :2 :S2, S20 :33 |  |
| --- | --- | --- |
|  | : |  |
|  | :D/E :•2YE |  |

Transport by sea

| Limited quantities (IMDG) | :1 L |  |  |
| --- | --- | --- | --- |
| Excepted quantities (IMDG) | :E2 |  |  |
| Packing instructions (IMDG) | :P001 |  |  |
| IBC packing instructions (IMDG) | :IBC02 |  |  |
| Tank instructions (IMDG) | :T4 |  |  |
| Tank special provisions (IMDG) | :TP1 |  |  |
| 06-08-2021 (Revision date) |  | EN (English) | 8/11 |

ACETONE EXTRA PURE

Safety Data Sheet 
according to Regulation (EC) No. 1907/2006 (REACH) with its amendment Regulation (EU) 2015/830

| EmS-No. (Fire) EmS-No. (Spillage) Stowage category (IMDG) Flash point (IMDG) MFAG-No | :F-E :S-D :E :-20°C to -18°C c.c. :127 |
| --- | --- |

Air transport 
PCA Excepted quantities (IATA) 	:E2 
PCA Limited quantities (IATA) 	:Y341 
PCA limited quantity max net quantity (IATA) 	:1L 
PCA packing instructions (IATA) 
	:353 PCA max net quantity (IATA) 	:5L 
CAO packing instructions (IATA) 	:364 
CAO max net quantity (IATA) 	:60L 
ERG code (IATA) 	:3H

Inland waterway transport 
Classification code (ADN) 	:F1 
Limited quantities (ADN) 	:1 L 
Excepted quantities (ADN) 	:E2 
Carriage permitted (ADN) 
	:T Equipment required (ADN) 	:PP, EX, A 
Ventilation (ADN) 	:VE01 
Number of blue cones/lights (ADN) 	:1

Rail transport 
Classification code (RID) 	:F1 
Limited quantities (RID) 	:1L 
Excepted quantities (RID) 	:E2 
Packing instructions (RID) 
	:P001, IBC02, R001 Mixed packing provisions (RID) 	:MP19 
Portable tank and bulk container instructions (RID) 	:T4 
Portable tank and bulk container special provisions 	:TP1 
(RID)

| Tank codes for RID tanks (RID) Transport category (RID) Colis express (express parcels) (RID) Hazard identification number (RID) | :LGBF :2 :CE7 :33 |
| --- | --- |

14.7. Transport in bulk according to Annex II of Marpol and the IBC Code

Not applicable

SECTION 15: Regulatory information

15.1. Safety, health and environmental regulations/legislation specific for the substance or mixture

15.1.1. EU-Regulations

| EU restriction list (REACH Annex XVII) |  |
| --- | --- |
| Reference code | Applicable on |
| 3(a) | ACETONE EXTRA PURE |
| 3(b) | ACETONE EXTRA PURE |
| 40. | ACETONE EXTRA PURE |

ACETONE EXTRA PURE is not on the REACH Candidate List 
ACETONE EXTRA PURE is not on the REACH Annex XIV List 
ACETONE EXTRA PURE is not subject to Regulation (EU) No 649/2012 of the European Parliament and of the Council of 4 july 2012 concerning the export and import of hazardous chemicals.

ACETONE EXTRA PURE is not subject to Regulation (EU) No 2019/1021 of the European Parliament and of the Council of 20 June 2019 on persistent organic pollutants

| 06-08-2021 (Revision date) | EN (English) | 9/11 |
| --- | --- | --- |

ACETONE EXTRA PURE 
Safety Data Sheet 
according to Regulation (EC) No. 1907/2006 (REACH) with its amendment Regulation (EU) 2015/830

15.1.2. National regulations

| France |  |
| --- | --- |
| Occupational diseases |  |
| Code | Description |
| RG 84 | Conditions caused by liquid organic solvents for professional use: saturated or unsaturated aliphatic or cyclic liquid hydrocarbons and mixtures thereof; liquid halogenated hydrocarbons; nitrated derivatives of aliphatic hydrocarbons; alcohols; glycols, glycol ethers; ketones; aldehydes; aliphatic and cyclic ethers, including tetrahydrofuran; esters; dimethylformamide and dimethylacetamine; acetonitrile and propionitrile; pyridine; dimethylsulfone and dimethylsulfoxide |

Germany 
Water hazard class (WGK) 	:WGK 1, Slightly hazardous to water (Classification according to AwSV; ID No. 6)

Hazardous Incident Ordinance (12. BImSchV) Netherlands

:Is not subject of the Hazardous Incident Ordinance (12. BImSchV)

SZW-lijst van kankerverwekkende stoffen 	:The substance is not listed SZW-lijst van mutagene stoffen 	:The substance is not listed NIET-limitatieve lijst van voor de voortplanting 	:The substance is not listed giftige stoffen – Borstvoeding 
NIET-limitatieve lijst van voor de voortplanting 
	:The substance is not listed giftige stoffen – Vruchtbaarheid 
NIET-limitatieve lijst van voor de voortplanting 	:The substance is not listed giftige stoffen – Ontwikkeling

Denmark

| Classification remarks Danish National Regulations | :Emergency management guidelines for the storage of flammable liquids must be followed :Young people below the age of 18 years are not allowed to use the product |
| --- | --- |

15.2. Chemical safety assessment

No chemical safety assessment has been carried out

SECTION 16: Other information Abbreviations and acronyms. ADN; European Agreement concerning the International Carriage of Dangerous Goods by Inland Waterways. ADR; European Agreement concerning the International Carriage of Dangerous Goods by Road. ATE; Acute Toxicity Estimate. BCF; Bioconcentration factor. BLV; Biological limit value. BOD; Biochemical oxygen demand (BOD). COD; Chemical oxygen demand (COD). DMEL; Derived Minimal Effect level. DNEL; Derived-No Effect Level. EC-No.; European Community number. EC50; Median effective concentration. EN; European Standard. IARC; International Agency for Research on Cancer. IATA; International Air Transport Association. IMDG; International Maritime Dangerous Goods. LC50; Median lethal concentration.

| 06-08-2021 (Revision date) | EN (English) | 10/11 |
| --- | --- | --- |

ACETONE EXTRA PURE 
Safety Data Sheet 
according to Regulation (EC) No. 1907/2006 (REACH) with its amendment Regulation (EU) 2015/830

| Abbreviations and acronyms |  |
| --- | --- |
| LD50 | Median lethal dose |
| LOAEL | Lowest Observed Adverse Effect Level |
| NOAEC | No-Observed Adverse Effect Concentration |
| NOAEL | No-Observed Adverse Effect Level |
| NOEC | No-Observed Effect Concentration |
| OECD | Organisation for Economic Co-operation and Development |
| OEL | Occupational Exposure Limit |
| PBT | Persistent Bioaccumulative Toxic |
| PNEC | Predicted No-Effect Concentration |
| RID | Regulations concerning the International Carriage of Dangerous Goods by Rail |
| SDS | Safety Data Sheet |
| STP | Sewage treatment plant |
| ThOD | Theoretical oxygen demand (ThOD) |
| TLM | Median Tolerance Limit |
| VOC | Volatile Organic Compounds |
| CAS-No. | Chemical Abstract Service number |
| N.O.S. | Not Otherwise Specified |
| vPvB | Very Persistent and Very Bioaccumulative |
| ED | Endocrine disrupting properties |

| Full text of H- and EUH-statements |  |
| --- | --- |
| Eye Irrit. 2 | Serious eye damage/eye irritation, Category 2 |
| Flam. Liq. 2 | Flammable liquids, Category 2 |
| STOT SE 3 | Specific target organ toxicity — Single exposure, Category 3, Narcosis |
| H225 | Highly flammable liquid and vapour. |
| H319 | Causes serious eye irritation. |
| H336 | May cause drowsiness or dizziness. |
| EUH066 | Repeated exposure may cause skin dryness or cracking. |

Safety Data Sheet (SDS), EU

This information is based on our current knowledge and is intended to describe the product for the purposes of health, safety and environmental requirements only. It should not therefore be construed as guaranteeing any specific property of the product.

| 06-08-2021 (Revision date) | EN (English) | 11/11 |
| --- | --- | --- |