MATERIAL SAFETY DATA SHEET

(MSDS)

PRODUCT NAME- ACETIC ACID

CAS No.: 64-19-7

| SECTION 1: Identification of the substance/mixture and of the company/undertaking |  |  |
| --- | --- | --- |
| 1.1. Product identifier |  |  |
| Product form | : | Substance |
| Product name | : | ACETIC ACID |
| Type of product | : | ACID |
| Product Grades (Applicable) | : | AR/ACS, LR, HPLC,CERTIFIED DRY |
| Product Code | : | 700001M500 |
|  |  | 700001L2D5 |
|  |  | 600001M500 |
|  |  | 600001L2D5 |
|  |  | 800001M500 |
|  |  | 900001L001 |
| CAS No | : | 64-19-7 |
| Molecular Formula | : | C2H4O2 |
| Molecular Weight (g/mol) | : | 60.05 |
| 1.2. Relevant identified uses of the substance or mixture and uses advised against |  |  |
| 1.2.1. Relevant identified uses | : | Industrial; For professional use only: Laboratory chemicals Manufacture of substances Solvents |
| 1.2.2. Uses advised against | : | No additional information available |
| 1.3. Details of the supplier of the safety data sheet |  |  |
| RYZE CHEMIE |  |  |
| Office No.13, Plot No.C-39 A |  |  |
| Gami Industrial Park, TTC Industrial Area, |  |  |
| Thane, Maharashtra-400705 |  |  |
| www.ryzechemie.com |  |  |
| 1.4. Emergency Telephone Number |  |  |
| +91- 99-3006-02-02 | : |  |
| SECTION 2: Hazards Identification |  |  |
| 2.1. Classification of the substance or mixture |  |  |
| Classification according to Regulation (EC) No. 1272/2008 [CLP] |  |  |
| Flammable liquids, Category 3 | H226 |  |
| Acute toxicity (oral), Category 4 | H302 |  |
| Acute toxicity (dermal), Category 4 | H312 |  |
| Acute toxicity (inhal.), Category 4 | H332 |  |
| Serious eye damage/eye irritation, | H319 |  |
| Category 2 |  |  |
| Full text of classification categories and H statements : see section 16 |  |  |
| Classification according to Directive 67/548/EEC or 1999/45/EC |  |  |
| F; R11 |  |  |
| Xi; R36 |  |  |
| R67 |  |  |
| Full text of R-phrases: see section 16 |  |  |
| Adverse physicochemical, human health and environmental effects |  |  |
| No additional information available |  |  |
| 2.2. Label Elements |  |  |
| Labelling according to Regulation (EC) No. 1272/2008 [CLP] |  |  |
| Hazard pictograms (CLP) | : |  |
|  |  | GHS02 GHS07 |
| Signal word (CLP) | : | Danger |
| Hazard statements (CLP) | : | H226 - Flammable liquid and vapour. |
|  |  | H314 - Causes severe skin burns and eye damage. |
| Precautionary statements (CLP) | : | P280 - Wear protective clothing, eye protection, face |
|  |  | protection, protective gloves. |
|  |  | P305+P351+P338 - IF IN EYES: Rinse cautiously with |
|  |  | water for several minutes.Remove |
|  |  | contact lenses,if present and easy to do. Continue rinsing. |
|  |  | P310 - Immediately call a doctor. |
| 2.3. Other hazards |  |  |
| Contains no PBT/vPvB substances ≥ 0.1% assessed in accordance with REACH Annex XIII |  |  |
| SECTION 3: Composition/information on ingredients |  |  |
| 3.1. Substance |  |  |
| Name | : | ACETIC ACID GLACIAL FOR HPLC |
| CAS No | : | 64-19-7 |
| EC-No. | : | 200-580-7 |
| EC Index-No. | : | 607-002-00-6 |
| 3.2. Mixture |  |  |
| Not applicable |  |  |
| SECTION 4: : First aid measures |  |  |
| 4.1. Description of first aid measures |  |  |
| First-aid measures general | : | Call a physician immediately |
| First-aid measures after inhalation | : | Remove person to fresh air and keep comfortable for |
|  |  | breathing. Immediately call a POISON CENTER/doctor. |
| First-aid measures after skin contact | : | Take off immediately all contaminated clothing.Rinse skin |
|  |  | with water/shower.Call a physician immediately. |
| First-aid measures after eye contact | : | Rinse cautiously with water for several minutes.Remove |
|  |  | contact lenses,if present and easy to do Continue rinsing. |
|  |  | Immediately call a POISON CENTER/doctor. |
| First-aid measures after ingestion | : | Rinse mouth. Do NOT induce vomiting. |
|  |  | Call a poison center or a doctor if you feel unwell. |
| 4.2. Most important symptoms and effects, both acute and delayed |  |  |
| Symptoms/effects after inhalation | : | Causes severe skin burns and eye damage. |
| Symptoms/effects after skin contact | : | Burns. |
| Symptoms/effects after eye contact | : | Causes serious eye irritation. Serious damage to eyes. |
| Symptoms/effects after ingestion | : | Burns. |
| 4.3. Indication of any immediate medical attention and special treatment needed |  |  |
| Treat symptomatically. |  |  |
| SECTION 5: Firefighting measures |  |  |
| 5.1. Extinguishing media |  |  |
| Suitable extinguishing media | : | Dry chemical powder, alcohol-resistant foam, |
|  |  | carbon dioxide (CO2). Water spray. Dry powder. Foam. |
|  |  | Carbon dioxide. |
| Unsuitable extinguishing media | : | Do not use a heavy water stream |
| 5.2. Special hazards arising from the substance or mixture |  |  |
| Fire hazard | : | Flammable liquid and vapour |
| Explosion hazard | : | May form flammable/explosive vapour-air mixture. |
| Hazardous decomposition products | : | Toxic fumes may be released. |
| in case of fire |  |  |
| 5.3. Advice for firefighters |  |  |
| Protection during firefighting | : | Do not attempt to take action without suitable protective |
|  |  | equipment. Self-contained breathing apparatus. |
|  |  | Complete protective clothing. |
| SECTION 6: Accidental release measures |  |  |
| 6.1. Personal precautions, protective equipment and emergency procedures |  |  |
| General measures | : | Remove ignition sources. Use special care to avoid |
|  |  | static electric charges. No open flames. No smoking. |
| 6.1.1. For non-emergency personnel |  |  |
| Emergency procedures | : | Ventilate spillage area. Evacuate unnecessary personnel. |
|  |  | No open flames, no sparks, and no smoking. |
|  |  | Avoid contact with skin and eyes. Do not breathe |
|  |  | Avoid breathing dust, fume, gas, mist, spray, vapours. |
| 6.1.2. For emergency responders |  |  |
| Protective equipment | : | Do not attempt to take action without suitable protective |
|  |  | equipment.Use personal protective equipment as required |
|  |  | For further information refer to section 8: |
|  |  | "Exposure controls/personal protection". |
| Emergency procedures | : | Stop release. |
| 6.2. Environmental precautions |  |  |
| Avoid release to the environment. |  |  |
| 6.3. Methods and material for containment and cleaning up |  |  |
| Methods for cleaning up | : | Take up liquid spill into absorbent material. Collect |
|  |  | spillage. On land, sweep or shovel into suitable containers. |
|  |  | Soak up spills with inert solids,such as clay or diatomaceous |
|  |  | earth as soon as possible. Notify authorities if product |
|  |  | enters sewers or public waters. |
| Other information | : | Dispose of materials or solid residues at an authorized site. |
| 6.4. Reference to other sections |  |  |
| For further information refer to section 13. |  |  |
| SECTION 7: Handling and storage |  |  |
| 7.1. Precautions for safe handling |  |  |
| Additional hazards when processed | : | Handle empty containers with care because residual |
|  |  | vapours are flammable. |
| Precautions for safe handling | : | Ensure good ventilation of the work station. Avoid contact |
|  |  | with skin, eyes and clothing. Do not breathe vapours. Keep |
|  |  | away from heat, hot surfaces, sparks, open flames and |
|  |  | other ignition sources.No smoking. Ground/bond container |
|  |  | and receiving equipment. Use only non-sparking tools. |
|  |  | Take precautionary measures against static discharge. |
|  |  | Flammable vapours may accumulate in the container. Use |
|  |  | explosion-proof equipment. Wear personal protective |
|  |  | equipment. Avoid contact with skin and eyes. Do not |
|  |  | breathe dust/fume/gas/mist/vapours/spray. |
| Hygiene measures | : | Wash hands and other exposed areas with mild soap and |
|  |  | water before eating,drinking or smoking and when leaving |
|  |  | work.Wash contaminated clothing before reuse.Do not |
|  |  | eat,drink or smoke when using this product. |
|  |  | Always wash hands after handling the product. |
| 7.2. Conditions for safe storage, including any incompatibilities |  |  |
| Technical measures | : | Comply with applicable regulations. Ground/bond |
|  |  | container and receiving equipment. |
| Storage conditions | : | Keep container tightly closed. Store in a well- |
|  |  | ventilated place. Keep cool. Store locked up. |
| Incompatible materials | : | Heat sources. |
| 7.3. Specific end use(s) |  |  |
| No additional information available |  |  |
| SECTION 8: Exposure controls/personal protection |  |  |
| 8.1. Control parameters |  |  |
| 8.1.1 National occupational exposure and biological limit values |  |  |
| No additional information available |  |  |
| 8.1.2. Recommended monitoring procedures |  |  |
| No additional information available |  |  |
| 8.1.3. Air contaminants formed |  |  |
| No additional information available |  |  |
| 8.1.4. DNEL and PNEC |  |  |
| No additional information available |  |  |
| 8.1.5. Control banding |  |  |
| No additional information available |  |  |
| 8.2. Exposure controls |  |  |
| Hand protection | : | Protective gloves |
| Eye protection | : | Chemical goggles or safety glasses |
| Skin and body protection | : | Wear suitable protective clothing |
| Respiratory protection |  | Wear appropriate mask |
| SECTION 9: Physical and chemical properties |  |  |
| 9.1. Information on basic physical and chemical properties |  |  |
| Physical state | : | Liquid |
| Appearance | : | Clear liquid. |
| Molecular mass | : | 60.05 g/mol |
| Colour | : | Colourless. |
| Odour | : | strong, vinegar-like. |
| Odour threshold | : | No data available |
| pH | : | 2.4 (1.0 M solution) |
| pH solution concentration | : | NA |
| Relative evaporation rate | : | 0.97 |
| Melting point | : | 16.2 °C |
| Freezing point | : | No data available |
| Boiling point | : | 117 – 118 °C |
| Flash point | : | 2 °C |
| Auto-ignition temperature | : | 485 °C |
| Decomposition temperature | : | No data available |
| Flammability (solid, gas) | : | Flammable liquid and vapour,Not applicable |
| Vapour pressure | : | 15.2 hPa at 20°C |
| Relative vapour density at 20 °C | : | 2.1 |
| Relative density | : | No data available |
| Density | : | 1.049 g/cm³ |
| Relative gas density | : | NA |
| Solubility | : | Water: Miscible in water |
| Partition coefficient n-octanol/water (Log Pow) | : | -0.17 |
| Viscosity, kinematic | : | 1.163 mm²/s |
| Viscosity, dynamic | : | 1.22 cP |
| Explosive properties | : | No data available |
| Oxidising properties | : | No data available |
| Explosive limits | : | 0.04 – 0.199 vol % |
| 9.2. Other information |  |  |
| No additional information available |  |  |
| SECTION 10: Stability and reactivity |  |  |
| 10.1. Reactivity |  |  |
| Thermal decomposition generates : Corrosive vapours. Flammable liquid and vapour |  |  |
| 10.2. Chemical Stability |  |  |
| Flammable liquid and vapour. May form flammable/explosive vapour-air mixture. |  |  |
| 10.3. Possibility of hazardous reactions |  |  |
| No dangerous reactions known under normal conditions of use. |  |  |
| 10.4. Conditions to avoid |  |  |
| Open flame. Overheating. Direct sunlight. Heat. Sparks. Avoid contact with hot surfaces. |  |  |
| No flames, no sparks. Eliminate all sources of ignition. |  |  |
| 10.5. Incompatible materials |  |  |
| No additional information available |  |  |
| 10.6. Hazardous decomposition products |  |  |
| May release flammable gases. Thermal decomposition generates : Corrosive vapours. |  |  |
| SECTION 11: Toxicological information |  |  |
| 11.1. Information on toxicological effects |  |  |
| Acute toxicity (oral) | : | Not classified |
| Acute toxicity (dermal) | : | Not classified |
| Acute toxicity (inhalation) | : | Not classified |
| Skin corrosion/irritation | : | Causes severe skin burns. |
|  |  | pH: 2.4 (1.0 M solution) |
| Serious eye damage/irritation | : | Assumed to cause serious eye damage |
|  |  | pH: 2.4 (1.0 M solution) |
| Respiratory or skin sensitisation | : | Not classified |
| Germ cell mutagenicity | : | Not classified |
| Carcinogenicity | : | Not classified |
| Reproductive toxicity | : | Not classified |
| STOT-single exposure | : | Not classified |
| STOT-repeated exposure | : | Not classified |
| Aspiration hazard | : | Not classified |
| Potential adverse human health | : | NA |
| effects and symptoms |  |  |
| SECTION 12: Ecological information |  |  |
| 12.1. Toxicity |  |  |
| Ecology - general | : | Before neutralisation, the product may represent a |
|  |  | danger to aquatic organisms. |
| Hazardous to the aquatic environment, | : | Not classified |
| short–term(acute) |  |  |
| Hazardous to the aquatic environment, | : | Not classified |
| long–term (chronic) |  |  |
| 12.2. Persistence and degradability |  |  |
| No additional information available |  |  |
| 12.3. Bioaccumulative potential |  |  |
| ACETIC ACID GLACIAL FOR HPLC (64-19-7) |  |  |
| Partition coefficient n-octanol/water (Log Pow) | : | 0.17 |
| 12.4. Mobility in soil |  |  |
| No additional information available |  |  |
| 12.5. Results of PBT and vPvB assessment |  |  |
| No additional information available |  |  |
| 12.6. Other adverse effects |  |  |
| No additional information available |  |  |
| SECTION 13: Disposal considerations |  |  |
| 13.1. Waste treatment methods |  |  |
| Waste treatment methods | : | Dispose of contents/container in accordance with licensed |
|  |  | collector’s sorting instructions. |
| Product/Packaging disposal recommendations |  | Dispose of contents/container to hazardous or special |
|  |  | l waste collection point, in accordance with local, regional, |
|  |  | national and/or international regulation. |
| Additional information | : | Handle empty containers with care because residual |
|  |  | vapours are flammable. Flammable vapours may |
|  |  | accumulate in the container. |
| SECTION 14: Transport information |  |  |
| In accordance with ADR / IMDG / IATA / ADN / RID |  |  |
| 14.1. UN number |  |  |
| UN-No. (ADR) | : | UN 2789 |
| UN-No. (IMDG) | : | UN 2789 |
| UN-No.(IATA) | : | UN 2789 |
| UN-No.(ADN) | : | UN 2789 |
| UN-No. (RID) | : | UN 2789 |
| 14.2. UN proper shipping name |  |  |
| Proper Shipping Name (ADR) | : | ACETIC ACID, GLACIAL |
| Proper Shipping Name (IMDG) | : | ACETIC ACID, GLACIAL |
| Proper Shipping Name (IATA) | : | acetic acid,glacial |
| Proper Shipping Name (ADN) | : | ACETIC ACID, GLACIAL |
| Proper Shipping Name (RID) | : | ACETIC ACID, GLACIAL |
| Transport document description (ADR) | : | UN 2789 ACETIC ACID, GLACIAL, 8 (3), II, (D/E) |
| Transport document description (IMDG) | : | UN 2789 ACETIC ACID, GLACIAL, 8 (3), II |
| Transport document description (IATA) | : | UN 2789 Acetic acid, glacial, 8 (3), II |
| Transport document description (ADN) | : | UN 2789 ACETIC ACID, GLACIAL, 8 (3), II |
| Transport document description (RID) | : | UN 2789 ACETIC ACID, GLACIAL, 8 (3), II |
| 14.3. Transport hazard class(es) |  |  |
| ADR |  |  |
| Transport hazard class(es) (ADR) | : | 8 (3) |
| Danger labels (ADR) | : | 8, 3. |
| IMDG |  |  |
| Transport hazard class(es) (IMDG) | : | 8 (3) |
| Danger labels (IMDG) | : | 8, 3 |
| IATA |  |  |
| Transport hazard class(es) (IATA) | : | 8 (3) |
| Hazard labels (IATA) | : | 8, 3 |
| ADN |  |  |
| Transport hazard class(es) (ADN) | : | 8 (3) |
| Hazard labels (ADN) | : | 8, 3 |
| RID |  |  |
| Transport hazard class(es) (RID) | : | 8 (3) |
| Hazard labels (RID) | : | 8, 3 |
| 14.4. Packing group |  |  |
| Packing group (ADR) | : | II |
| Packing group (IMDG) | : | II |
| Packing group (IATA) | : | II |
| Packing group (ADN) | : | II |
| Packing group (RID) | : | II |
| 14.5. Environmental hazards |  |  |
| Dangerous for the environment | : | No |
| Marine pollutant | : | No |
| Other information | : | No supplementary information available |
| 14.6. Special precautions for user |  |  |
| Overland transport | : |  |
| Classification code (ADR) | : | CF1 |
| Limited quantities (ADR) | : | 1L |
| Excepted quantities (ADR) | : | E2 |
| Packing instructions (ADR) | : | P001, IBC02 |
| Mixed packing provisions (ADR) | : | MP15 |
| Portable tank and bulk container instructions (ADR) | : | T7 |
| Portable tank and bulk container special provisions (ADR) | : | TP2 |
| Tank code (ADR) | : | L4BN |
| Vehicle for tank carriage | : | FL |
| Transport category (ADR) | : | 2 |
| Special provisions for carriage - Operation (ADR) | : | S2 |
| Hazard identification number (Kemler No.) | : | 83 |
| Orange plates | : |  |
| Tunnel restriction code (ADR) | : | D/E. |
| EAC code | : | •2P |
| - Transport by sea |  |  |
| Limited quantities (IMDG) | : | 1L |
| Excepted quantities (IMDG) | : | E2 |
| Packing instructions (IMDG) | : | P001 |
| IBC packing instructions (IMDG) | : | IBC02 |
| Tank instructions (IMDG) | : | T7 |
| Tank special provisions (IMDG) | : | TP2 |
| EmS-No. (Fire) | : | F-E |
| EmS-No. (Spillage) | : | S-C |
| Stowage category (IMDG) | : | A |
| Flash point (IMDG) | : | 2°C c.c. |
| Properties and observations (IMDG) | : | Colourless flammable liquid with a pungent odour. When |
|  |  | n pure, crystallizes below 16°C. Flashpoint: 40°C c.c. |
|  |  | Explosive limits: 4% to 17% Miscible with water |
|  |  | Corrosive to lead and most other metals. |
| MFAG-No | : | 132 |
| - Air transport |  |  |
| PCA Excepted quantities (IATA) | : | E2 |
| PCA Limited quantities (IATA) | : | Y840 |
| PCA limited quantity max net quantity (IATA) | : | 0.5L |
| PCA packing instructions (IATA) | : | 851 |
| PCA max net quantity (IATA) | : | 1L |
| CAO packing instructions (IATA) | : | 855 |
| CAO max net quantity (IATA) | : | 30L |
| ERG code (IATA) | : | 8F |
| - Inland waterway transport |  |  |
| Classification code (ADN) | : | CF1 |
| Limited quantities (ADN) | : | 1L |
| Excepted quantities (ADN) | : | E2 |
| Carriage permitted (ADN) | : | T |
| Equipment required (ADN) | : | PP, EP, EX, A |
| Ventilation (ADN) | : | VEO1 |
| Number of blue cones/lights (ADN) | : | 1 |
| - Rail transport |  |  |
| Classification code (RID) | : | CF1 |
| Limited quantities (RID) | : | 1L |
| Excepted quantities (RID) | : | E2 |
| Packing instructions (RID) | : | P001, IBC02 |
| Mixed packing provisions (RID) | : | MP15 |
| Portable tank and bulk container instructions (RID) | : | T7 |
| Portable tank and bulk container special provisions (RID) | : | TP2 |
| Tank codes for RID tanks (RID) | : | L4BN |
| Transport category (RID) | : | 2 |
| Colis express (express parcels) (RID) | : | CE6 |
| Hazard identification number (RID) | : | 83 |
| 14.7. Transport in bulk according to Annex II of MARPOL 73/78 and the IBC Code |  |  |
| Not applicable |  |  |
| SECTION 15: Regulatory information |  |  |
| 15.1. Safety, health and environmental regulations/legislation specific for the substance or mixture |  |  |
| 15.1.1. EU-Regulations |  |  |
| REACH Annex XVII (Restriction List) |  |  |
| REACH Annex XIV (Authorisation List) |  |  |
| ACETIC ACID GLACIAL FOR HPLC is not on the REACH Annex XIV List |  |  |
| REACH Candidate List (SVHC) |  |  |
| ACETIC ACID GLACIAL FOR HPLC is not on the REACH Candidate List |  |  |
| PIC Regulation (Prior Informed Consent) |  |  |
| ACETIC ACID GLACIAL FOR HPLC is not subject to Regulation (EU) No 649/2012 of the European |  |  |
| Parliament and of the Council of 4 july 2012 concerning the export and import |  |  |
| of hazardous chemicals. |  |  |
| POP Regulation (Persistent Organic Pollutants) |  |  |
| ACETIC ACID GLACIAL FOR HPLC is not subject to Regulation (EU) No 2019/1021 of the European |  |  |
| Parliament and of the Council of 20 June 2019 on persistent organic pollutants |  |  |
| Ozone Regulation (1005/2009) |  |  |
| ACETIC ACID GLACIAL is not subject to REGULATION (EU) No 1005/2009 OF THE EUROPEAN |  |  |
| PARLIAMENT AND OF THE COUNCIL of 16 September 2009 on substances |  |  |
| that deplete the ozone layer. |  |  |
| Explosives Precursors Regulation (2019/1148) |  |  |
| Contains no substance subject to Regulation (EU) 2019/1148 of the European Parliament |  |  |
| and of the Council of 20 June 2019 on the marketing and use of explosives precursors. |  |  |
| Drug Precursors Regulation (273/2004) |  |  |
| Contains no substance(s) listed on the Drug Precursors list(Regulation EC 273/2004 on drug precursors) |  |  |
| 15.1.2. National regulations |  |  |
| France |  |  |
| Germany |  |  |
| Water hazard class (WGK) : WGK 1, Slightly hazardous to water |  |  |
| (Classification according to AwSV; ID No. 93). |  |  |
| Hazardous Incident Ordinance (12. BImSchV) :Is not subject of the Hazardous Incident Ordinance |  |  |
| (12. BImSchV) |  |  |
| Netherlands |  |  |
| SZW-lijst van kankerverwekkende stoffen | : | The substance is not listed |
| SZW-lijst van mutagene stoffen | : | The substance is not listed |
| SZW-lijst van reprotoxische stoffen – | : | The substance is not listed |
| Borstvoeding |  |  |
| SZW-lijst van reprotoxische stoffen – | : | The substance is not listed |
| Vruchtbaarheid |  |  |
| SZW-lijst van reprotoxische stoffen – | : | The substance is not listed |
| Ontwikkeling |  |  |
| Denmark |  |  |
| Class for fire hazard | : | Class II-1 |
| Store unit | : | 5 liter |
| Classification remarks | : | R10 ; Emergency management guidelines for the storage |
|  |  | of flammable liquids must be followed. |
| Danish National Regulations | : | : Young people below the age of 18 years are not allowed |
|  |  | to use the product |
| Switzerland |  |  |
| Storage class (LK) | : | LK 3 - Flammable liquids |
| 15.2. Chemical safety assessment |  |  |
| No chemical safety assessment has been carried out |  |  |
| SECTION 16: Other information |  |  |
| Acute Tox. 4 (Dermal) |  | Acute toxicity (dermal), Category 4 |
| Acute Tox. 4 (Inhalation) |  | Acute toxicity (inhal.), Category 4 |
| Acute Tox. 4 (Oral) |  | Acute toxicity (oral), Category 4 |
| Eye Irrit. 2 |  | Serious eye damage/eye irritation, Category 2 |
| Flam. Liq. 2 |  | Flammable liquids, Category 2 |
| H225 |  | Highly flammable liquid and vapour. |
| H302 |  | Harmful if swallowed. |
| H312 |  | Harmful in contact with skin. |
| H319 |  | Causes serious eye irritation. |
| H332 |  | Harmful if inhaled |
| Safety Data Sheet (SDS), EU |  |  |
| This information is based on our current knowledge and is intended to describe the product for the |  |  |
| purposes of health, safety and environmental requirements only. It should not therefore be construed |  |  |
| as guaranteeing any specific property of the product. |  |  |